# METHOD OF STATEMENT

Date: 04th Sep 2023

Description of work

To unload 1 case from lorry using forklift, uncrate, detach machine from base, transport to door entrance, shift and position to designated location using rollers

Machine Spec

1 unit Microdrilling machine

Dim: 4212 x 1758 mm

Wt: 5, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/09/23

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay, ISS Plant

ST Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x low loader

1 x 10 ton forklift

Pallet truck/rollers

Metal sheet/canvas (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deployment of lorry loader and low loader to site

# Using 10 ton forklift to fork down case from lorry chassis to the ground

# Uncrate case and put aside

# Loosen all nuts form machine bed

# Forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175