# METHOD OF STATEMENT

Date: 13th Apr 2021

Description of work

To unload 3 cases of 1 unit machine from 1 x 40ft FR container using forklift, uncrate, detach machine from base, transport to door entrance, shift and position to designated location

Machine Spec

EMCO HT45

Case 1: 365 x 245 x 246 cm   /   G/N Wt: 4, 800 / 3900 kg

Case 2: 356 x 210 x 205 cm   /   G/N Wt: 2, 300 / 1, 700 kg

Case 3: 230 x 225 x 238 cm   /   G/N Wt: 1, 000 / 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBA

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

Advanced Material Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10 ton forklift

Pallet truck/rollers

Metal sheet/canvas

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deploy lorry loader at the designated location

# Using 10 ton forklift to fork down cases from container to the ground

# Uncrate case

# Forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers/pallet truck to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175