# METHOD OF STATEMENT

Date: 19th Dec 2025

Description of work

Uncrate case, using forklift to fork and detach machine from base, shift and position into factory premise

Machine Spec

1 unit Mitutoyo CMM / Crysta-Apex

Dim: 315 x 244 x 269 cm

Wt: 7, 639 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/12/25

Time of Operation

09:00 am

Location of Operation

80 Tuas West Dr

Innovex Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10ton forklift

Hand jack

Woodblock

Rollers/pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at designated unloading area

# Uncrate machine case

# Use forklift to fork up and detach machine from base

# Load up machine onto machine stand

# Transport machine to near door entrance

# Using rollers/pallet truck to shift machine into factory premise at ground floor

# Position machine at the designated location

# Unload machine from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175