# METHOD OF STATEMENT

Date: 06th Oct 2022

Description of work

To unload 18 cases of 4 unit machine from 2 x 40ft FR and 5 x 40ft HQ container using forklift, uncrate, detach machine from base, shift and position into factory premise at ground floor

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

11-12/10/22

Time of Operation

09:40 am

Location of Operation

16 Seletar Aerospace Crescent

Dolphin Engineering Pte Ltd

Equipment Use

633 lorry loader

1 x 16 ton forklift

1 x 3ton forklift

Pallet truck/rollers

Metal sheet/canvas (to lay out for floor protection

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in the operation

# Job Scope

# Deployment of lorry loader and manpower to site

# Unload necessary equipment needed for the job

# Using 16 ton forklift to fork down cases from container

# Unload case to the ground

# Uncrate case and put aside

# Loosen all nuts attached to machine

# 16ton forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175