# METHOD OF STATEMENT

Date: 24th Jan 2025

Description of work

To unload 1 case from 1 x 20ft container, uncrate, using forklift to detach machine from base, transport to loading bay, using roller to shift and position to 4th floor via cargo lift

Spec

LP-500 Vacuum Pre-Melt Coater

Case Dim: 370 x 230 x 226 cm

N/G Wt: 2, 430 kg / 2, 980 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/02/25

Time of Operation

09:30 am

Location of Operation

18 Woodlands Loop

Edmund Optics Singapore Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 4.5ton forklift

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Metal sheet/canvas (to lay out for floor epoxy protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader back to back with container

# Using crane to pull out case from container

# Load case onto lorry chassis

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case and put aside

# Forklift to fork up and detach machine from base

# Transport and unload machine at loading bay

# Using roller to transport machine to 4th floor via cargo lift

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175