# METHOD OF STATEMENT

Date: 07th Oct 2025

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down machine from lorry chassis to the ground, shift and position into factory premise at ground floor

Machine Spec

1 unit Induction Furnace with accessories

Refer to attach packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 09-10/10/25

Time of Operation

Est: 11:30 am

Location of Operation

249 Jalan Boon Lay

ST Engineering Advanced Material Pte Ltd

Equipment Use

3 x 633 lorry loader

1 x 4.5ton forklift

Wire/webbing sling

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Canvas, metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up lorry crane with outriggers fully extended and resting on steel plate/timber mats as base plate

# Rigger to insert sling to crane hook

# Secured sling to rigging point of forklift

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry chassis to the ground

# Transport and unload machine at door entrance

# Using rollers/pallet truck to shift machine into factory premise at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175