# METHOD OF STATEMENT

Date: 11th June 2025

Description of work

To unload 24 cases of 1 units machine from 1 x 40ft HQ and 1 x 20GP container using forklift, uncrate, detach machine from base, transport to door entrance, shift and position to designated location using pallet truck

Machine Spec

ESP machine

1 x 40ft HQ container

Total packages: 17

Gross Wt: 7, 260 kg

1 x 20ft GP container

Total packages: 7

Gross Wt: 1, 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/06/25

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

ST Engineering Advanced Material Engineering Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x 7 ton forklift

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Metal sheet/canvas (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deployment of lorry loader to site

# Using 7ton forklift to fork down cases from container

# Unload case to the ground

# Uncrate case and put aside

# Forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using pallet truck to transport machine into production area at ground floor

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175