# METHOD OF STATEMENT

Date: 16th Sep 2024

Description of work

To fork down machineries from lorry using forklift, unload at loading bay, shift and position to designated location using rollers/pallet truck

Machine Spec

Refer to attach list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

2 day job

12/10/24

13/10/24

Time of Operation

09:45 am

Location of Operation

8 Commonwealth Lane #01-01

Acot Precision Engineering Pte Ltd

Equipment Use

4 x 633 lorry loader

1 x 7 ton forklift

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Metal sheet/canvas (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift driver

# Job Scope

# Deployment of lorry loader to site

# Using 7 ton forklift to fork down machine from lorry chassis

# Transport to loading bay

# Unload machine onto woodblock

# Using rollers/pallet truck to transport machine into tenant premise at ground floor

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Repeat process till all machine have been move in

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175