# METHOD OF STATEMENT

Date: 02nd Sep 2021

Description of work

1) Using forklift to transport 1 unit machine into factory premise, unload at unloading area, shift and position to location using pallet truck

2) To shift out 1 unit machine to loading area, forklift to fork up machine, transport to outside factory and load onto lorry chassis

Machine Spec

Move in

1 unit Cut AM500

Dim: 3240 x 1930 x 2530 mm / Wt: 3, 775 kg

Move out

1 unit Cut AM500

Dim: 3240 x 1930 x 2530 mm / Wt: 3, 775 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/09/21

Time of Operation

Est: 12:30 pm

Location of Operation

1 Pandan Crescent

Emerson Asia Pacific Pte Ltd

Equipment Use

1 x 633 lorry loader

Crowbar/woodblock

Pallet truck/rollers

1 x 4.5 ton forklift

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift operator

# Job Scope

# 1)

# Deploy lorry loader at outside of customer factory premise

# Using forklift to fork down machine from lorry chassis

# Forklift to transport machine into inside factory premise

# Unload machine near roller shutter door

# Using pallet truck to transport machine into production area

# Position machine at the designated location

# 2)

# Shift out old machine from production to loading area

# Using forklift to fork up machine

# Transport machine to outside of factory premise

# Unload machine onto lorry chassis

# Secured machine with strapping belt to other location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175