# METHOD OF STATEMENT

Date: 15th Sep 2020

Description of work

Using forklift to fork down case from lorry chassis to the ground, uncrate, to detach machine from base, shift and position into factory premise at ground floor using pallet truck

Machine Spec

1 unit Alzmetall Column Drilling machine

Dim: 129 x 93 x 200 cm

Wt: 268 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/09/20

Time of Operation

Location of Operation

1 Pandan Crescent

Emerson Asia Pacific Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 x 2.5 ton forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift operator

# Job Scope

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case

# Forklift to fork up and detach machine from base

# Transport and unload machine at unloading bay

# Using pallet truck to transport machine into factory premise at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175