# METHOD OF STATEMENT

Date: 08th Aug 2025

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down case from lorry chassis, uncrate, shift and position into factory premise

Machine Spec

2 cases of 1 unit used InspeXio SMX-225CT

Case 1: 244 x 157 x 219 cm / N, G wt: 3, 040 kg / 3, 382 kg

Case 2: 200 x 151 x 155 cm / N, G wt: 440 kg / 598 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/09/25

Time of Operation

09:30 am

Location of Operation

1 Tuas Rd

Murata Electronics (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x 4.5ton forklift

Crowbar/woodblock

Wire/webbing sling

Pallet truck

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up crane with outrigger fully extended and resting on timber mats as base plate

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case

# Use forklift to fork up and detach machine from base

# Transport machine and unload at near door entrance

# Using pallet truck to shift machine into factory premise

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175