# METHOD OF STATEMENT

Date: 25th June 2024

Description of work

Lorry crane to hoist down 1 unit forklift from lorry chassis, using forklift to fork down machine from lorry to the ground, shift and position into factory premise at ground floor using pallet truck

Machine Spec

1 unit Fiber Laser Cutting machine

Wt: 1, 358 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

28/06/24

Time of Operation

Est:

12:00 pm

Location of Operation

100 Woodlands Ave 7

3M Singapore Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x forklift

Wire/webbing sling

Pallet truck

Crowbar

Woodblock

Canvas/PVC plate (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry chassis to near ground level

# Transport and unload machine at loading bay

# Using pallet truck to transport machine into factory premise at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175