# METHOD OF STATEMENT

Date: 12th Aug 2025

Description of work

To shift out 1 unit machine from 2nd floor production to ground floor loading area via cargo lift, forklift to fork up onto lorry chassis & deliver to customer.

Machine Spec

1 unit Xray machine  (Dim: 145 x 190 x 230cm/ Wt: 2,800kgs)

+ Control console (Dim: 140 x 80 x 150cm)

+ Chiller & Transformer

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

11 Calshot Rd

Equipment Use

2 x lorry crane

1 x forklift

Pallet truck

Wood block/ Crowbar

Shackle

Wire sling

Canvass/ metal sheet (to lay out for floor protection

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# - Deploy lorry loader at designated unloading area

# - Hoist down forklift from lorry chassis to the ground

# - Insert pallet truck under machine base

# - Shift out machine from production room at 2nd floor to ground floor loading area via cargo lift

# - At ground floor loading area, using forklift to fork up machine

# - Load up machine onto lorry chassis

# - Secured machine with strapping belt prior to moving out of customer premise

# - Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# - Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175