# METHOD OF STATEMENT

Date: 05th Aug 2025

Description of work

Lorry loader to hoist down 1 unit forklift from lorry chassis, using forklift to fork down machine to the ground, shift and position into factory premise

Machine Spec

1 unit Xray machine

Dim: 145 x 190 x 230 cm

Control console: 140 x 80 x 150 cm

2 x rack stand

1 x small chiller

1 x transformer

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

11 Calshot Rd

Equipment Use

1 x 633 lorry loader

1 x 3ton forklift

Webbing sling

Pallet truck

Crowbar/woodblock

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up of lorry crane with outriggers fully extended and resting on timber mats as base plate

# Rigger to insert sling to crane hook and secured to rigging point of forklift

# Hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry chassis

# Transport and unload machine near door entrance

# Using pallet truck to shift into factory premise

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175