# METHOD OF STATEMENT

Date: 13th Aug 2025

Description of work

Lorry crane to hoist down forklift to the ground, using forklift to fork up and detach machine from base, shift and position into workshop premise at ground floor

Machine Spec

3 cases of 1 unit machine

Case 1: 295 x 122 x 178 cm / Wt: 730 kg

Case 2: 366 x 122 x 183 cm / Wt: 1, 746 kg

Case 3: 92 x 92 x 102 cm / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

28-29/08/25

Time of Operation

10:00 am

Location of Operation

13 New Punggol Rd

SIT

Equipment Use

1 x 633 lorry loader

1 x 3ton battery forklift

Crowbar/woodblock

Webbing sling

Pallet truck

Canvas/PVC plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up crane with outrigger fully extended and resting on timber mats as base plate

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Forklift to fork down case from lorry chassis to the ground

# Uncrate case and put aside

# Using forklift to fork and detach machine from base

# Transport and unload machine at door entrance

# Using pallet truck to shift machine into workshop

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175