# METHOD OF STATEMENT

Date: 11th Apr 2024

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down crate from forwarder lorry to the ground and shift to the balcony for uncrating, shift and position equipment into factory premise.

3 crate

Crate 1: 216 x 166 x 203 cm

Crate 2: 212 x 173 x 155 cm

Crate 3: 117 x 127 x 139 cm

Total wt: 1, 957 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/04/24

Time of Operation

09:45 am

Location of Operation

20 Tuas South Ave 14

Dyson Facility

Equipment Use

1 x 633 lorry loader

1 x 3ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry loader to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case from forwarder lorry

# Uncrate case and put aside

# Using forklift to fork up and detach equipment from base

# Transport equipment near door entrance

# Fork up equipment about 1m high

# Unload equipment onto building balcony

# Using pallet truck to shift equipment to location

# Position equipment at the designated location

# Unload equipment from pallet truck and insert levelling pad onto equipment bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175