# METHOD OF STATEMENT

Date: 12th Aug 2024

Description of work

To shift out 1 unit machine to loading area, fork up onto lorry chassis and deliver to another location

Machine Spec

1 unit machine

Dim: 106 x 106 cm

Wt: 680 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/08/24

Time of Operation

10:00 am

Location of Operation

206 Bedok South Ave 1

Certis Group

Equipment Use

1 x lorry loader

1 x manual lifter

Webbing sling

Pallet truck

Canvas/PVC plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Hoist down manual lifter from lorry chassis to the ground

# Use manual lifter to lift down machine from platform

# To shift out machine from production to loading area using pallet truck

# Certis personnel will assist to fork machine

# Load up machine onto lorry chassis

# Secured machine with strapping belt prior to being transport out of premise

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175