# METHOD OF STATEMENT

Date: 17th Dec 2025

Description of work

Lorry crane to hoist down forklift from lorry, using forklift to fork down machine from lorry chassis, shift and position into production at ground floor

Machine Spec

1 unit machine

Dim: 1250 x 1400 x 2100 mm

Wt: 900 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/12/25

Time of Operation

10:00 am

Location of Operation

23 Loyang Way

GE Aviation

Equipment Use

1-2 x lorry loader

1 x 2.5ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at the designated unloading area

# Set up crane with outrigger fully extended and resting on timber mats as base plate

# Hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry to the ground

# Transport and unload machine at door entrance

# Using pallet truck to transport machine into production at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175