# METHOD OF STATEMENT

Date: 13th July 2024

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down machine from lorry to the ground, shift and position into factory premise at ground floor

Machine Spec

1 set Automation machine (dismantle to 2 parts)

Wt: 1, 500 kg each parts

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

10/09/24

Time of Operation

12:00 pm

Location of Operation

249 Jalan Boon Lay

ST Engineering

Equipment Use

2 x 633 lorry loader

1 x 3ton battery forklift

Wire/webbing sling

Rollers/pallet truck

Crowbar/woodblock

Canvas, metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Setting up of lorry crane with outriggers fully extended and resting on steel plate/timber mats as base plate

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry chassis

# Transport machine to near door entrance

# Using pallet truck to shift machine into factory premise at ground floor

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175