# METHOD OF STATEMENT

Date: 15th Oct 2025

Description of work

Using lorry loader and forklift to unload cases from lorry chassis, uncrate, shift and position into factory premise at ground floor

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est 2-3 days job

27-29/10/25

Time of Operation

10:00 am

Location of Operation

25 A.M.K St 65

Sanden International (S) Pte Ltd

Equipment Use

3-4 x 633 lorry loader

1 x 3ton battery forklift

Webbing sling

Pallet truck

Crowbar

Woodblock

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up of lorry crane with outriggers fully extended and resting on timber mats as base plate

# Rigger to insert sling to crane hook and secured to rigging point of forklift

# Hoist down forklift from lorry chassis to the ground

# Using forklift and lorry crane to unload cases from lorry chassis to the ground

# Uncrate case and put aside

# Fork/hoist up to detach from base

# Unload machine near door entrance

# Using pallet truck to shift into factory premise

# Position at the designated location

# Battery forklift to assist customer engineer install gantry inside factory

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175