# METHOD OF STATEMENT

Date: 30th Oct 2024

Description of work

1) To hoist down 1 unit forklift from lorry chassis, using forklift to fork down 1 unit machine from lorry chassis to the ground, shift and position into workshop at ground floor

2) To shift out 1 unit machine from workshop to loading area, using forklift to fork up onto lorry chassis

Machine Spec

Move in

1 unit Twinhorn VF500

Dim: 240 x 152 x 237 cm / Wt: 2, 600 kg

Move out

1 unit Hardinge VCM

Wt: 2, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/11/24

Time of Operation

09:45 am

Location of Operation

3 Science Dr 2, Blk S15

NUS

Equipment Use

2 x 633 lorry loader

1 x 4.5ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry chassis

# Lower down machine to near ground level

# Transport machine to near door entrance

# Unload machine to the ground

# Using pallet truck to shift machine into factory premise

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# To shift out 1 unit old machine to loading area

# Using forklift to fork up machine onto lorry chassis

# Transport and deliver to new location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175