# METHOD OF STATEMENT

Date: 26th Apr 2021

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down cases to the ground, uncrate, detach machine from base, using pallet truck to shift and position to location

Spec

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/05/2021

Time of Operation

10:00 am

Location of Operation

30 Biopolis Street, Matrix Building

Research Support Center #B2-14

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

Pallet truck

Canvas sheet (to lay out for floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Hoist down forklift from lorry chassis

# Using forklift to fork down cases from forwarder lorry

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at unloading bay

# Using pallet truck to transport machine to location

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175