# METHOD OF STATEMENT

Date: 14th Oct 2024

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down 2 cases of 1 unit machine from lorry chassis to the ground, uncrate, shift and position into factory premise.

Machine Spec

1 unit LH87-1000 CMM

Case 1: 212 x 165 x 132 cm / Wt: 2, 182 kg

Case 2: 260 x 174 x 125 cm / Wt: 344 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/11/24

Time of Operation

09:45 am

Location of Operation

30 Pioneer Crescent #06-18/19

Dyson Operation Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 3ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry loader to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case and put aside

# Using forklift to fork up and detach machine from base

# Transport machine to near door entrance

# Unload machine to the ground

# Using pallet truck to shift machine into factory premise

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175