# METHOD OF STATEMENT

Date: 06th Dec 2024

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down machine and accessories to the ground, shift and position into factory premise at 2nd floor via cargo lift

Machine Spec

1 unit Laser machine

Machine: 155 x 150 x 193 cm Wt: 1, 200 kg

1 unit Robot arm

1 unit Robat arm stand

1 unit Robot arm

2 unit Trolley

1 unit Robot controller

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

13/12/24

Time of Operation

Est:

13:00 pm

Location of Operation

32 Loyang Way

Lincstech Circuit Singapore Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x 3.5ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up of lorry crane with outriggers fully extended and resting on timber mats as base plate

# Rigger to insert sling to crane hook

# Secured sling to rigging point of forklift

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine and accessories from lorry chassis

# Transport and unload at near door entrance

# Lorry loader to hoist up Robot arm onto stand

# Using pallet truck to shift into factory premise at 2nd floor via cargo lift

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175