# METHOD OF STATEMENT

Date: 13th Mar 2024

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down 6 cases from forwarder lorry to the ground, uncrate, shift and position to 3rd floor via cargo lift

Machine Spec

6 cases of 1 unit Farsoon HT403P Laser Sintering machine

Case Dim: 2900 x 1810 x 2495 mm / Wt: 3, 000 kg

Case Dim: 1700 x 850 x 1600 mm / Wt: 350 kg

Case Dim: 960 x 830 x 530 mm / Wt: 100 kg

Case Dim: 510 x 660 x 1000 mm / Wt: 100 kg

Case Dim: 950 x 1350 x 2050 mm / Wt: 230 kg

Case Dim: 940 x 945 x 2275 mm / Wt: 357 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/03/24

Time of Operation

09:45 am

Location of Operation

5 Cleantech Loop, Cleantech 2

Simtech

Equipment Use

1 x 633 lorry loader

1 x 4.5ton forklift

Wire sling

Pallet truck

Crowbar

Woodblock

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case from forwarder lorry to the ground

# Uncrate case and put aside

# Using forklift to fork up and detach machine from base

# Transport and unload machine at loading bay

# Using pallet truck to transport machine to 3rd floor via cargo lift

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175