# METHOD OF STATEMENT

Date: 11th Mar 2025

Description of work

Lorry crane to hoist down case to the ground, using forklift to fork up and detach machine from base, shift and position into factory premise at ground floor

Machine Spec

Agility S796 CMM machine

Dim: 220 x 190 x 240 cm

Wt: 2, 900 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15-18/03/25

Time of Operation

09:30 am

Location of Operation

5 Tuas Dr 2

TOS

Equipment Use

1 x 633 lorry loader

1 x 3ton battery forklift

Crowbar/woodblock

Webbing sling

Pallet truck

Canvas/PVC plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up crane with outrigger fully extended and resting on timber mats as base plate

# Lorry crane to hoist down case from lorry chassis to the ground

# Using forklift to fork and detach machine from base

# Transport forklift with machine into furnace room

# Lift up machine and let it sit onto the machine stand

# Using pallet truck to shift machine into CMM room

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175