# METHOD OF STATEMENT

Date: 04th Aug 2021

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down cases to the ground, uncrate, detach machine from base, using pallet truck to shift and position to location

Spec

Case 1: 195 x 150 x 223 cm / Wt: 1, 174 kg

Case 2: 107 x 106 x 201 cm / Wt: 347 kg

Case 3: 122 x 82 x 167 cm / Wt: 261 kg

Case 4: 122 x 82 x 167 cm / Wt: 221 kg

Case 5: 149 x 99 x 96 cm / Wt: 154 kg

Box 6: 82 x 122 x 76 cm / Wt: 60 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

50 Nanyang Ave N4, 1-B4-08

Nanyang Technological University, School Of Materials Science & Engineering

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

Pallet truck

Canvas/plywood (to lay out for floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Hoist down forklift from lorry chassis

# Using forklift to fork down cases from forwarder lorry

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at unloading bay

# Using pallet truck to transport machine to location

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175