# METHOD OF STATEMENT

Date: 19th Sep 2025

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down case from forwarder lorry, uncrate, shift and position into factory premise

Machine Spec

1 unit Test Stand

Dim: 387 x 257 x 247 cm

Wt: 3, 648 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/09/25

Time of Operation

09:45 am

Location of Operation

503 Airport Rd

ST Engineering Aerospace Engines Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 4.5ton forklift

Hand jack/crowbar

Woodblock

Wire/webbing sling

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up crane with outrigger fully extended and resting on timber mats as base plate

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine case from forwarder lorry to the ground

# Uncrate case

# Use forklift to fork up and detach machine from base

# Transport machine to near door entrance

# Using pallet truck to shift machine into factory premise at ground floor

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175