# METHOD OF STATEMENT

Date: 01st Dec 2023

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down 3 pallet from lorry to the ground, shift and position into factory premise

Modula machine

Pallet 1: 2500 x 2300 x 2600 mm / Wt: 1, 200 kg

Pallet 2: 2000 x 1500 x 1000 mm / Wt: 900 kg

Pallet 3: 2000 x 1500 x 1000 mm / Wt: 900 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/12/23

Time of Operation

Afternoon schedule

Location of Operation

540 Airport Rd

ST Engineering Aerospace System Pte Ltd

Equipment Use

2-3 x 633 lorry loader

1 x 3ton forklift

Wire/webbing sling

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down pallet from lorry chassis

# Transport pallet to near door entrance

# Using pallet truck to shift pallet into factory premise

# Position pallet at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175