# METHOD OF STATEMENT

Date: 02nd Dec2024

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down 2 case, uncrate, shift and position into workshop at ground floor

Machine Spec

CNC machine

Case 1: 175 x 105 x 230 cm / Wt: 1, 300 kg

Case 2: 49 x 44 x 63 cm / Wt: 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

NUS, 9 Engineering Dr 1, Blk EA

Department of Mechanical Engineering

Equipment Use

2 x 633 lorry loader

1 x 3ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case

# Fork up and detach machine from base

# Transport machine to near door entrance

# Using pallet truck to shift machine into workshop

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175