# METHOD OF STATEMENT

Date: 29th Dec 2023

Description of work

To hoist down forklift from lorry chassis, using forklift to fork down cases to the ground, uncrate case, fork up and detach from base, transport and unload at door entrance, using pallet truck to shift and position at designated location

Spec

4 cases of 4 set Chip Conveyor

Dim: 239 x 112 x 158 cm

N/G Wt: 158/327 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/01/24

Time of Operation

09:45 am

Location of Operation

91 Tuas Ave 1

SMC Manufacturing (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x 3ton battery forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Metal plate/canvas (to lay out onto roadside and inside factory for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry loader to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down cases to the ground

# Uncrate case and put aside

# Forklift to fork up and detach conveyor from base

# Transport and unload conveyor at door entrance

# Using pallet truck to shift into production at ground floor

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175