# METHOD OF STATEMENT

Date: 13th Jan 2025

Description of work

To hoist down and unload machines and accessories from lorry chassis using lorry crane and forklift, shift and position into tenant premise

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

16/01/25 – 26/01/25

Time of Operation

13:00 pm

Location of Operation

Blk 211 Woodlands Ave 9 #04-80/81, Woodlands Spectrum 2

Index Aerospace Pte Ltd

Equipment Use

4-5 x lorry loader

1 x 4.5ton forklift

1 x 10ton forklift

Wire/webbing sling

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

# Lorry crane to hoist down machine from lorry chassis to the ground

# Using of forklift to fork down machine from lorry chassis

# Lower down machine to near ground level

# Transport machine to near door entrance

# Unload machine to the ground

# Using rollers/pallet truck to shift machine into factory premise

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175