# METHOD OF STATEMENT

Date: 26th Feb 2021

Description of work

To hoist down forklift from lorry chassis to the ground, forklift to detach machine from base, transport and unload at door entrance, shift and position into premise using pallet truck

Forklift Spec

1 unit forklift

Dim: 2.8m x 1m x 2m

Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

05/03/21

Time of Operation

09:30 am

Location of Operation

NTU, 71 Nanyang Drive

Singapore Institute of Mfg Technology

Equipment Use

1 x 633 lorry loader

Wire sling

Pallet truck

1 x 3 ton forklift

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Lorry loader to hoist down forklift from lorry chassis to the ground

# Uncrate case

# Fork up and detach machine from base

# Transport and unload machine at door entrance

# Using pallet truck to transport machine into premise at ground floor

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175