# METHOD OF STATEMENT

Date: 26th Feb 2021

Description of work

To hoist down 1 unit forklift, using forklift to fork down 4 cases to the ground, uncrate, detach machine from base, using pallet truck to shift and position to 9th floor via cargo lift

Machine Spec

Case 1: 98 x 98 x 160 cm / Wt: 235 kg

Case 1: 122 x 80 x 120 cm / Wt: 155 kg

Case 1: 98 x 98 x 160 cm / Wt: 285 kg

Case 1: 170 x 130 x 120 cm / Wt: 510 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

02/03/2021

Time of Operation

Est: 11:00 am

Location of Operation

NUS, 4 Science Drive

Department of Materials Science & Engineering, S9

Equipment Use

2 x 633 lorry loader

Wire/webbing sling

Pallet truck

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Hoist down forklift from lorry chassis

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at unloading bay

# Using pallet truck to transport machine to 9th floor via cargo lift

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175