# METHOD OF STATEMENT

Date: 05th July 2023

Description of work

To hoist down forklift from lorry chassis, using forklift fork down machine from lorry, shift and position into premise at ground floor

Machine Spec

1 unit WF-300P

Dim: 295 x 153 x 205 cm

Wt: 2, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

05/07/23

Time of Operation

12:00 pm

Location of Operation

Padang Singapore

Equipment Use

2 x 633 lorry loader

1 x 4.5ton forklift

Wire sling

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Hoist down forklift from lorry

# Using forklift to fork down machine from lorry chassis

# Transport into premise at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175