# METHOD OF STATEMENT

Date: 15th Aug 2021

Description of work

Using forklift to fork down cases to the ground, uncrate, detach machine from base, using rollers/ pallet truck to shift and position into production at ground floor, assist customer engineer to assemble machine

Machine Spec

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

09:45 am

Location of Operation

Grand Venture Technology

Equipment Use

633 lorry loader

Wire/webbing sling

Rollers/pallet truck

Metal plate/canvas (to lay out for floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy trailer at designated unloading area

# Using forklift to fork down case from lorry chassis

# Uncrate case

# Fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers to transport machine into production area

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175