# METHOD OF STATEMENT

Date: 19th Fed 2024

Description of work

To hoist down forklift from lorry chassis, using forklift to fork down machine from lorry chassis.

Shift & position into premises

Spec

Case 1 (Dim: 136(L) x 123(W) x 153(H) cm/ G. Wt: 672kgs / N.Wt:560kgs)

Case 2 (Dim: 73(L) x 77(W) x 53(H) cm / G. Wt: 21kgs / N.Wt: 7kgs)

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/02/24

Time of Operation

11: 00am

Location of Operation

11A Tuas Ave 20

Denka Advantech P/L

Equipment Use

1 x 633 lorry loader

1 x 2.5 forklift

Wire/webbing sling

Pallet truck

Metal plate for floor protection

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry loader to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down machine from lorry chassis

# Transport & position at designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175