# METHOD OF STATEMENT

Date: 18th Aug 2020

Description of work

Lorry loader to hoist down forklift, using forklift to unload 2 cases from forwarder lorry, uncrate, shift position to premise at ground floor

Machine Spec

Case 1: 126 x 146 x 215 cm / Wt: 584 kg (to uncrate, shift and position to location)

Case 2: 213 x 146 x 216 cm / Wt: 1, 256kg (without uncrate, shift and position to location)

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/08/2020   –   To unload, shift and position to location

21/08/2020   -   To shift out for packing, after packing, load onto forwarder lorry

Time of Operation

Afternoon schedule

Location of Operation

3 Bedok South Rd

Panasonic Industrial Devices Singapore

Equipment Use

1 x 633 lorry loader – PALFINGER PK50002EH

1 set wire/webbing sling

Pallet truck

1 x 3 ton forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

# Job Scope

# 19/08/2020

# Deploy lorry loader at the designated location

# Hoist down forklift from lorry chassis

# Using forklift to fork down 2 cases from forwarder lorry

# Uncrate case no 1

# Fork up and detach machine from base

# Transport and unload machine at entrance door

# Using pallet truck to transport machine into premise at ground floor

# Position machine at the designated location

# 21/08/2020

# To shift out machine from customer premise to loading area

# Using forklift to fork up machine and load onto wooden base

# Pack to form a case

# After packing, fork up case and load onto forwarder lorry

# When job complete, hoist up forklift and load onto lorry chassis

# Team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175