# METHOD OF STATEMENT

Date: 16th Nov 2021

Description of work

To hoist down forklift from lorry chassis, using forklift fork down vacuum pump, ramp up slope, unload at level ground, shift and position into pump room

Spec

1 unit VADS 1500 Vacuum Pump

Dim: 170 x 150 x 180 cm

Wt: 1, 400 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

29/11/21

Time of Operation

11:30 am

Location of Operation

133 Tuas South Ave 3

Ciba Vision Asian Manufacturing & Logistics Ltd

Equipment Use

1 x 633 PALFINGER lorry loader

1 x 3ton forklift

Wire sling

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Hoist down forklift from lorry

# Forklift to fork down vacuum pump from lorry chassis

# Fork vacuum pump up slope

# Unload vacuum pump onto level ground

# Using pallet truck to shift into pump room

# Position vacuum pump at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175