# METHOD OF STATEMENT

Date: 02nd Oct 2023

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down case to the ground, uncrate case, fork up and detach machine from base, transport and unload at door entrance, using pallet truck to shift and position into CMM room at ground floor

Machine Spec

1 unit Global Touch 12.15.10 CMM

Case Dim: 3418 x 2178 x 2549 mm

Wt: 3, 850 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/10/23

Time of Operation

09:45 am

Location of Operation

2 Pioneer Sector 3

GE Repair Solutions Singapore

Equipment Use

2 x 633 lorry loader

1 x 4.5ton forklift

Wire/webbing sling

Pallet truck

Hand jack/woodblock

Canvas, PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet, ear plug)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case

# Uncrate case and put aside

# Forklift to fork up and detach machine from base

# Transport machine into factory premise

# Unload machine at door entrance

# Using pallet truck to shift machine into CMM room at ground floor

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad onto machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175