# METHOD OF STATEMENT

Date: 07th Nov 2023

Description of work

To hoist down forklift from lorry chassis, using forklift to detach machine from base, transport to door entrance, shift and position to designated location

Machine Spec

1 unit Paragon CNC Grinding

Wt: 4, 120 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/11/23

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

STE

Equipment Use

1 x 633 lorry loader

Wire sling

Pallet truck/rollers

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated location

# Hoist down forklift from lorry chassis to the ground

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers/pallet truck to transport machine into production at ground floor

# Position machine at the designated location

# Unload machine from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175