# METHOD OF STATEMENT

Date: 10th Mar 2020

Description of work

To unload 1 unit machine from 1 x 20ft GP container, uncrate, using forklift to detach machine from base, shift and position to 3rd floor via cargo lift

Machine Spec

1 unit Laser Tec 30 SLM

Dim: 2673 x 925 x 2015 mm

Wt: 1, 900 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBA

Location of Operation

3 Cleantech Loop #01-01

Advanced Remanufacturing & Technology Centre (ARTC)

Equipment Use

1 x lorry loader – PALFINGER PK50002EH

Wire sling

Pallet truck

1 x 3 ton forklift

Canvas, plywood, metal plate (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at the designated location (back to back with container)

# Using crane to pull out machine case from container and load onto lorry chassis

# Fork down case to the ground

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at unloading bay

# Using rollers/pallet truck to transport machine to 3rd floor via cargo lift

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175