# METHOD OF STATEMENT

Date: 19th July 2019

Description of work

To unload 1 unit machine with accessories from 1 x 40ft OT container, uncrate, using forklift to detach machine from base, transport to door entrance, using pallet truck/rollers to shift and position to designated location

Machine Spec

1 unit BEAM Machine with accessories

Dim: 2275 x 1850 x 2560 mm / Wt: 4, 400 kgs

Dim: 2500 x 1850 x 2560 mm / Wt: 2, 500 kgs

Dim: 900 x 900 x 1500 mm / Wt: 320 kg

Dim: 650 x 800 x 1300 mm / Wt: 560 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date/Time of Operation

Estimate: 30/07/2019   -   09:45 am

Location of Operation

3 Cleantech Loop #01-01

Advanced Remanufacturing & Technology Centre (ARTC)

Equipment Use

1 x lorry loader – PALFINGER PK50002EH

Wire sling

Pallet truck/rollers

1 x 4.5 ton forklift

Canvas, plywood, metal plate (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at the designated location

# Open up container canvas

# Pull out machine case from container and load onto lorry chassis

# Hoist down case from lorry chassis to the ground

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Lay out canvas, metal plate onto floor for protection

# Using rollers/pallet truck to transport machine into production

# Position machine at the designated location

# Unload machine from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175