# METHOD OF STATEMENT

Date: 17th Feb 2022

Description of work

To unload 4 cases from lorry chassis using forklift, uncrate, shift and position into factory premise at ground floor

Machine Spec

1 x Form Talysurf PGI 2540

1 x Talyrond 395 Roundness

Control panel & cylinder

Drybox & 2 unit cabinet

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/03/22

Time of Operation

09:45 am

Location of Operation

8 Cleantech Loop

Blk 1B, 01-22A, CT3

Equipment Use

2 x lorry loader

Wire/webbing sling

Pallet truck

1 x 3 ton forklift

Canvas, metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Hoist down forklift from lorry chassis

# Using forklift to fork down cases from lorry chassis to the ground

# Uncrate case and put aside

# Forklift to fork up and detach machine from base

# Transport and unload machine near shutter door opening

# Using pallet truck to transport machine into factory premise

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175