# METHOD OF STATEMENT

Date: 19th Feb 2021

Description of work

To hoist down forklift from lorry chassis to the ground, forklift to unload case, uncrate, detach machine from base, shift and position to 2nd floor via cargo lift

Spec

1 x forklift

Dim: 2.8m x 1m x 2m

Wt: 3, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

Location of Operation

Blk 5002 A.M.K Ave 5 #02-06

TMX Precision Tooling Pte Ltd

Equipment Use

2 x 633 lorry loader

Wire sling

Pallet truck

1 x 3.5 ton forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Lorry loader to hoist down forklift from lorry chassis to the ground

# Using forklift to fork down case

# Uncrate case

# Fork up and detach machine from base

# Load back machine onto lorry chassis

# Using pallet truck to transport machine to 2nd floor via cargo lift

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175