# METHOD OF STATEMENT

Date: 04th Jan 2023

Description of work

To unload 1 unit machine from lorry chassis using forklift, shift and position to 2nd floor via cargo lift

Machine Spec

1 unit machine

Dim: 140 x 94 x 176 cm

Wt: 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/01/23

Time of Operation

11:30 am

Location of Operation

3 Cleantech Loop

Cleantech 2B

Equipment Use

1 x lorry loader

Pallet truck

1 x 2.5 ton forklift

Canvas, pvc sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift operator

# Job Scope

# Deploy lorry loader at the designated unloading area

# Using forklift to fork down machine from lorry chassis to the ground

# Transport and unload machine at unloading bay

# Using pallet truck to transport machine to 2nd floor via cargo lift

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175