# METHOD OF STATEMENT

Date: 19th Oct 2020

Description of work

To unload 4 cases of 2 unit machine from lorry chassis to the ground, uncrate, using forklift to detach machine from base, transport and unload at door entrance, shift and position into premise using pallet truck/rollers

Machine Spec

3 cases of 1 unit DMP FLEX 350/3D Printer

Case 1: 263 x 195 x 293 cm   /   G. Wt: 5, 060 kgs

Case 2: 272 x 166 x 237 cm   /   G. Wt: 1, 394 kgs

Case 3: 168 x 123 x 194 cm   /   G. Wt: 87 kgs

1 case of Sieving Station

Case 1: 148 x 109 x 242 cm

G. Wt: 450 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

09:45 am

Location of Operation

NTU Valley Block

Equipment Use

1 x 633 lorry loader

Wire sling

Pallet truck/rollers

1 x 4.5 ton forklift

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated location

# Hoist down case from container to the ground

# Uncrate case

# Using forklift to fork up and detach machine from base

# Transport and unload machine at door entrance

# Using rollers/pallet truck to transport machine into production at ground floor

# Position machine at the designated location

# Unload machine from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175