# METHOD OF STATEMENT

Date: 15th Nov 2019

Description of work

To unload 10 cases of machine and accessories from 2 x container, uncrate, using pallet truck to shift and position to location

Machine Spec

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/11/2019

Time of Operation

09:45 am

Location of Operation

73 Nanyang Drive

Singapore Institute of Manufacturing Technology

Equipment Use

1 x 633 lorry loader

1 set wire/webbing sling

Pallet truck/rollers

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gears with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated location (back to back with container)

# Hoist down forklift from lorry chassis

# Crane to pull out case from container onto lorry chassis

# Using forklift to fork down case from lorry chassis to the ground

# Uncrate case

# Fork up and detach machine from base

# Transport and unload machine at entrance door

# Using pallet truck/rollers to transport machine into premise

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175