# METHOD OF STATEMENT

Date: 19th Sep 2024

Description of work

To fork down 1 unit machine from lorry chassis, shift and position into factory premise

Machine Spec

1 unit machine

Dim: 106 x 106 cm

Wt: 680 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/09/24

Time of Operation

Estimate:

11:30 am

Location of Operation

206 Bedok South Ave 1

Certis Group

Equipment Use

1 x lorry loader

Pallet truck

1 x reach stacker (to be operated by Certis personnel)

Canvas/PVC plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Deploy lorry loader at the designated unloading area

# Certis personnel will assist to fork down machine from lorry chassis

# To transport machine into production using pallet truck

# Fork up machine onto platform

# Shift and position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175