# METHOD OF STATEMENT

Date: 14th Mar 2025

Description of work

To fork down and unload machine and accessories from lorry chassis using forklift, shift and position into workshop at ground floor

Machine Spec

Mill E 500U

Dim: 2650 x 3300 x 2450 mm

Wt: 6, 030 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27-28/03/25

Time of Operation

13:00 pm

Location of Operation

Singapore Institute of Technology

13 New Punggol Rd

Equipment Use

2 x lorry loader

1 x 10ton forklift

1 x low loader

Wire sling

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

# Deploy lorry crane at designated unloading area

# Using of 10ton forklift to fork down machine from lorry chassis

# Lower down machine to near ground level

# Transport machine to near door entrance

# Insert rollers under machine bed

# Using rollers to transport machine into workshop premise

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175